Introduction

In this lab we Implemented a MIPS single cycle processor with new functionality. This was done by adding new hardware to a base version provided to us.

Tasks which were completed in lab:

- Custom official draft of extended datapath and control unit.

- Custom official draft of extended MIPS microarchitecture.

- Control unit truth table.

- Design and verify MIPS processor extension design.

  - Waveform software simulation

  - FPGA hardware simulation

Design Methodology

The basis of this lab was the single-cycle MIPS processor provided in previous labs. This processor was capable of the ADD, SUB, AND, OR, SLT, LW, SW, BEQ, J, and ADDI functions. The content of this lab required that we add more to the architecture to provide functionality for the MULTU, MFHI, MFLO, JR, and JAL. Machine code was provided to the us for testing purposes. With this code, we were able to verify proper functionality through a visual testbench as well as implementing the design on the Nexys4 DDR FPGA. In the following sections, we have provided images and tables that demonstrate the validation process. These figures demonstrate both the expected results as well as the actual outcome. This lab taught us how to validate a processor design using a FPGA board. This allows us a deeper understanding of processor functionality.

Figure 1. Extended Datapath and Control Unit

Figure 2. Control Unit

Table 1. Control Unit Output Table

|  | Input |  | Output |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instruction | opcode | Funct | RegWrite | RegDst [1:0] | AluSrc | Branch | MemWrite | MemtoReg [1:0] | Jump | JReturn | selJal | writehilo | Aluop | AluControl |
| add | 000000 | 100000 | 1 | 01 | 0 | 0 | 0 | 00 | 0 | 0 | 0 | 0 | 10 | 010 |
| sub | 000000 | 100010 | 1 | 01 | 0 | 0 | 0 | 00 | 0 | 0 | 0 | 0 | 10 | 110 |
| and | 000000 | 100100 | 1 | 01 | 0 | 0 | 0 | 00 | 0 | 0 | 0 | 0 | 10 | 000 |
| or | 000000 | 100101 | 1 | 01 | 0 | 0 | 0 | 00 | 0 | 0 | 0 | 0 | 10 | 001 |
| slt | 000000 | 101010 | 1 | 01 | 0 | 0 | 0 | 00 | 0 | 0 | 0 | 0 | 10 | 111 |
| lw | 100011 | x | 1 | 00 | 1 | 0 | 0 | 01 | 0 | 0 | 0 | 0 | 00 | 010 |
| sw | 101011 | x | 0 | xx | 1 | 0 | 1 | xx | 0 | 0 | 0 | 0 | 00 | 010 |
| beq | 000100 | x | 0 | xx | 0 | 1 | 0 | xx | 0 | 0 | 0 | 0 | 01 | 110 |
| addi | 001000 | x | 1 | 00 | 1 | 0 | 0 | 00 | 0 | 0 | 0 | 0 | 00 | 010 |
| j | 000100 | x | 0 | xx | x | x | 0 | xx | 1 | 0 | 0 | 0 | xx | xxx |
| jr | 000000 | 001000 | 0 | xx | x | 0 | 0 | 00 | 0 | 1 | 0 | 0 | 10 | 010 |
| jal | 000011 | x | 1 | 10 | 0 | 0 | 0 | xx | 1 | 0 | 1 | 0 | xx | xxx |
| slr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ssl |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Simulation Test Plan

The simulation was the result of the testbench used to validate the architecture design built in this lab. The testbench ran the provided machine code through our design and the simulation output the results. By careful examination, we could validate inputs and outputs of the new functions included in our design. We could also validate the contents of specific registers at any given time in the program runtime.

Figure 3. Processor Testbench Validation

FPGA Test Plan

The hardware implementation of this design was performed on the Nexys4 DDR FPGA board. The 7-Segment LEDs were used to output the contents of various registers determined by the DIP-switches below them. For validation purposes of this lab, we were interested in the contents of the wd_dm register which was signified by writing a hex value of 10 to the switches.

Figure 4. Processor Hardware Verification

Figure 5. Extended Processor Hardware Verification

Conclusion

In summary, this lab was meant to familiarize the students with microprocessor design. We were provided a single cycle MIPS processor and tasked with adding to the microarchitecture to allow for functionality of additional functions. Once the design was completed, it was tested through visual validation of a waveform as well as hardware implementation using a Nexys4 DDR FPGA board. The final functionality performed according to the expected results, and overall our understanding of the MIPS processor was greatly improved.

Source Code

test_code.asm

main:

# addi $sp, $0, 48        not for SPIM

addi $a0, $0, 4 # set arg

jal factorial # compute the factorial

add $s0, $v0, $0 # move result into $s0

j end

factorial:

addi $sp, $sp, -8   # make room on stack

sw $a0, 4($sp) # store $a0

sw $ra, 0($sp) # store $ra

addi $t0, $0, 2 # $t0 = 2

slt $t0, $a0, $t0 # a <= 1 ?

beq $t0, $0, else # no - goto else

addi $v0, $0, 1 # yes - return 1

addi $sp, $sp, 8 # restore $sp

jr $ra # return

else:

addi $a0, $a0, -1   # n = n - 1

jal factorial # recursive call

lw $ra, 0($sp) # restore $ra

lw $a0, 4($sp) # restore $a0

addi $sp, $sp, 8 # restore $sp

multu $a0, $v0 # n * factorial(n-1)

mflo $v0 # mv result into $v0

jr $ra

end:

Mips_fpga.xdc

set_property -dict { PACKAGE_PIN E3    IOSTANDARD LVCMOS33 } [get_ports { clk }];

create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports {clk}];

set_property -dict { PACKAGE_PIN N17   IOSTANDARD LVCMOS33 } [get_ports { button }];

set_property -dict { PACKAGE_PIN P17   IOSTANDARD LVCMOS33 } [get_ports { rst }];

set_property -dict { PACKAGE_PIN J15   IOSTANDARD LVCMOS33 } [get_ports { switches[0] }];

set_property -dict { PACKAGE_PIN L16   IOSTANDARD LVCMOS33 } [get_ports { switches[1] }];

set_property -dict { PACKAGE_PIN M13   IOSTANDARD LVCMOS33 } [get_ports { switches[2] }];

set_property -dict { PACKAGE_PIN R15   IOSTANDARD LVCMOS33 } [get_ports { switches[3] }];

set_property -dict { PACKAGE_PIN R17   IOSTANDARD LVCMOS33 } [get_ports { switches[4] }];

set_property -dict { PACKAGE_PIN T18   IOSTANDARD LVCMOS33 } [get_ports { switches[5] }];

set_property -dict { PACKAGE_PIN U18   IOSTANDARD LVCMOS33 } [get_ports { switches[6] }];

set_property -dict { PACKAGE_PIN R13   IOSTANDARD LVCMOS33 } [get_ports { switches[7] }];

set_property -dict { PACKAGE_PIN H17   IOSTANDARD LVCMOS33 } [get_ports { we_dm }];

set_property -dict { PACKAGE_PIN K15   IOSTANDARD LVCMOS33 } [get_ports { sink_bit }];

set_property -dict { PACKAGE_PIN K13   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[0] }];

set_property -dict { PACKAGE_PIN K16   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[1] }];

set_property -dict { PACKAGE_PIN P15   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[2] }];

set_property -dict { PACKAGE_PIN L18   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[3] }];

set_property -dict { PACKAGE_PIN R10   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[4] }];

set_property -dict { PACKAGE_PIN T11   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[5] }];

set_property -dict { PACKAGE_PIN T10   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[6] }];

set_property -dict { PACKAGE_PIN H15   IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[7] }];

set_property -dict { PACKAGE_PIN J17   IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[0] }];

set_property -dict { PACKAGE_PIN J18   IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[1] }];

set_property -dict { PACKAGE_PIN T9    IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[2] }];

tb.v

module tb_mips_top;

reg         clk, rst;

wire        we_dm;

wire [31:0] pc_current, instr, alu_out, wd_dm, rd_dm;

mips_top DUT (clk, rst, we_dm, pc_current, instr, alu_out, wd_dm, rd_dm);

task tick; begin #5 clk = 1; #5 clk = 0; end endtask

task rest; begin #5 rst = 1; #5 rst = 0; end endtask

initial begin

rest;

while(pc_current != 32'h54) tick;

$display("passed");

$finish;

end

endmodule

cu.v

module controlunit

(input zero, [5:0] opcode, funct, output pc_src, jump, we_reg, alu_src, we_dm, JReturn, writeHilo, selJal, [1:0] dm2reg, reg_dst, [2:0] alu_ctrl);

wire [1:0] alu_op;

assign pc_src = branch & zero;

maindec md (opcode, funct, branch, jump, writeHilo, selJal, we_reg, alu_src, we_dm, dm2reg, reg_dst, alu_op);

auxdec  ad (alu_op, funct, JReturn, alu_ctrl);

Endmodule

dp.v

module datapath

(input clk, rst, pc_src, jump, JReturn, writeHilo, selJal,  we_reg, alu_src, [1:0] dm2reg, reg_dst, [2:0] alu_ctrl, [4:0] ra3, [31:0] instr, rd_dm, output zero, [31:0] pc_current, alu_out, wd_dm, rd3);

wire [4:0]  rf_wa;

wire [31:0] pc_plus4, pc_pre, pc_next, sext_imm, ba, bta, jta, alu_pa, alu_pb, mulHi, mulLo, q_mulHi, q_mulLo, wd_rf, data_out, pc_jump, shamt_out, wd_final;

wire wd_sel;

assign wd_sel = (alu_ctrl == 3'b100 | alu_ctrl == 3'b101);

assign ba = {sext_imm[29:0], 2'b00};

assign jta = {pc_plus4[31:28], instr[25:0], 2'b00};

// --- PC Logic --- //

dreg       pc_reg     (clk, rst, pc_next, pc_current);

adder      pc_plus_4  (pc_current, 4, pc_plus4);

adder      pc_plus_br (pc_plus4, ba, bta);

mux2 #(32) pc_src_mux (pc_src, pc_plus4, bta, pc_pre);

mux2 #(32) pc_jmp_mux (jump, pc_pre, jta, pc_jump);

mux2 #(32) pc_jr_mux  (JReturn, pc_jump, wd_final, pc_next)

// --- RF Logic --- //

mux4 #(5)  rf_wa_mux  (reg_dst, instr[20:16], instr[15:11], 31, 0, rf_wa);

regfile    rf         (clk, we_reg, instr[25:21], instr[20:16], ra3, rf_wa, wd_final, alu_pa, wd_dm, rd3);

signext    se         (instr[15:0], sext_imm);

// --- ALU Logic --- //

mux2 #(32) alu_pb_mux (alu_src, wd_dm, sext_imm, alu_pb);

alu        alu        (alu_ctrl, alu_pa, alu_pb, zero, alu_out);

//--- MUL Logic ---//

mul        mulOp      (alu_pa, alu_pb, mulHi, mulLo);

en_dreg    mul_hi_reg (writeHilo, rst, mulHi, q_mulHi);

en_dreg    mul_lo_reg (writeHilo, rst,  mulLo, q_mulLo);

// --- SHIFT Logic --- //

shifter SHAMT (alu_ctrl, alu_pa, instr[10:6], shamt_out);

// --- MEM Logic --- //

mux4 #(32) rf_data_mux  (dm2reg, alu_out, rd_dm, q_mulHi, q_mulLo, data_out);

mux2 #(32) rf_wd_mux    (selJal, data_out, pc_plus4, wd_rf);

mux2 #(32) wd_sel_mux   (wd_sel, wd_rf, shamt_out, wd_final);

endmodule

cu_modules.v

module maindec

(input [5:0] opcode, [5:0] funct, output branch, jump, writeHilo, setJal, we_reg, alu_src, we_dm, [1:0] dm2reg, reg_dst, alu_op);

reg [12:0] ctrl;

assign {branch, jump, reg_dst, we_reg, alu_src, we_dm, dm2reg, alu_op, writeHilo, setJal} = ctrl;

always @ (opcode, funct)

begin

case (opcode)

6'b00_0000:

begin

if(funct == 6'b00_1000) //JumpReturn

ctrl = 13'b0_0_01_0_1_0_00_10_0_0;

else if(funct == 6'b01_1001)

ctrl = 13'b0_0_01_0_0_0_00_10_1_0; //MultU

else if(funct == 6'b01_0000)

ctrl = 13'b0_0_01_1_0_0_10_10_0_0; //MfHi

else if(funct == 6'b01_0010)

ctrl = 13'b0_0_01_1_0_0_11_10_0_0; //MfLo

else

ctrl = 13'b0_0_01_1_0_0_00_10_0_0; // R-type

end

6'b00_1000: ctrl = 13'b0_0_00_1_1_0_00_00_0_0; // ADDI

6'b00_0100: ctrl = 13'b1_0_00_0_0_0_00_01_0_0; // BEQ

6'b00_0010: ctrl = 13'b0_1_00_0_0_0_00_00_0_0; // J

6'b10_1011: ctrl = 13'b0_0_00_0_1_1_00_00_0_0; // SW

6'b10_0011: ctrl = 13'b0_0_00_1_1_0_01_00_0_0; // LW

6'b00_0011: ctrl = 13'b0_1_10_1_0_0_00_00_0_1; //JAL

default:    ctrl = 13'bx_x_xx_x_x_x_xx_xx_x_x;

endcase

end

endmodule

module auxdec

(input [1:0] alu_op, [5:0] funct, output Jreturn, [2:0] alu_ctrl);

reg [2:0] ctrl;

reg jReg;

assign {alu_ctrl} = ctrl;

assign {Jreturn} = jReg;

always @ (alu_op, funct)

begin

case (alu_op)

2'b00:

begin

ctrl = 3'b010;

jReg = 0;

end

2'b01:

begin

ctrl = 3'b110;

jReg = 0;

end

default: case (funct)

6'b10_0100:

begin

ctrl = 3'b000;

jReg = 0;

end

6'b10_0101:

begin

ctrl = 3'b001;

jReg = 0;

end

6'b10_0000:

begin

jReg = 0;

ctrl = 3'b010;

end

6'b10_0010:

begin

jReg = 0;

ctrl = 3'b110;

end

6'b10_1010:

begin

jReg = 0;

ctrl = 3'b111;

end

6'b00_1000:

begin

jReg = 1;

ctrl = 3'b011;

end

6'b01_1001:

begin

jReg = 0;

ctrl = 3'b000;

end

6'b01_0000:

begin

jReg = 0;

ctrl = 3'b000;

end

6'b01_0010:

begin

jReg = 0;

ctrl = 3'b000;

end

6'b00_0000:

begin

jReg = 0;

ctrl = 3'b100;

end

6'b00_0010:

begin

jReg = 0;

ctrl = 3'b101;

end

default:    ctrl = 3'bxxx;

endcase

endcase

end

endmodule

Memfile.dat

20040004

0C000004

00408020

08000015

23BDFFF8

AFA40004

AFBF0000

20080002

0088402A

10080003

20020001

23BD0008

03E00008

2084FFFF

0C000004

8FBF0000

8FA40004

23BD0008

00820019

00001012

03E00008

module mux2 #(parameter wide = 8)

(input sel, [wide-1:0] a, b, output [wide-1:0] y);

assign y = (sel) ? b : a;

endmodule

module mux4 #(parameter wide = 8)

(input [1:0] sel, [wide-1:0] a, b, c, d, output reg [wide-1:0] y);

always@ (sel, a, b, c, d)

begin

case(sel)

2'b00: y = a;

2'b01: y = b;

2'b10: y = c;

2'b11: y = d;

endcase

end

endmodule

module adder

(input [31:0] a, b, output [31:0] y);

assign y = a + b;

endmodule

module signext

(input [15:0] a, output [31:0] y);

assign y = {{16{a[15]}}, a};

endmodule

module alu

(input [2:0] op, [31:0] a, b, output zero, reg [31:0] y);

assign zero = (y == 0);

always @ (op, a, b)

begin

case (op)

3'b000: y = a & b;

3'b001: y = a | b;

3'b010: y = a + b;

3'b011: y = a + 0;

3'b110: y = a - b;

3'b111: y = (a < b) ? 1 : 0;

endcase

end

endmodule

module mul

(input [31:0] x, y, output reg [31:0] outHi, outLo);

reg [63:0] out;

always @ (x, y)

begin

out = x * y;

outHi = out[63:32];

outLo = out[31:0];

end

endmodule

module shifter #(parameter DATA_WIDTH = 32) (

input wire [2:0] enable_op,

input wire [DATA_WIDTH - 1:0] rd, shamt,

output reg [DATA_WIDTH - 1:0] data_out

);

always @(*) begin

case (enable_op)

3'b100: data_out = rd << shamt;

3'b101: data_out = rd >> shamt;

default: data_out = data_out;

endcase

end

endmodule

module dreg

(input clk, rst, [31:0] d, output reg [31:0] q);

always @ (posedge clk, posedge rst)

begin

if (rst) q <= 0;

else     q <= d;

end

endmodule

dp_modules.v

module en_dreg

(input en, rst, [31:0] d, output reg [31:0] q);

always @ (d, en, rst)

begin

if (rst) q <= 0;

else if(en)

q <= d;

else q <= q;

end

endmodule

module regfile

(input clk, we, [4:0] ra1, ra2, ra3, wa, [31:0] wd, output [31:0] rd1, rd2, rd3);

reg [31:0] rf [0:31];

integer n;

initial begin

for (n = 0; n < 32; n = n + 1) rf[n] = 32'h0;

end

always @ (posedge clk)

begin

if (we) rf[wa] <= wd;

end

assign rd1 = (ra1 == 0) ? 0 : rf[ra1];

assign rd2 = (ra2 == 0) ? 0 : rf[ra2];

assign rd3 = (ra3 == 0) ? 0 : rf[ra3];

endmodule

Mips.v

module mips_top

(input clk, rst, output we_dm, [31:0] pc_current, instr, alu_out, wd_dm, rd_dm);

wire [31:0] DONT_USE;

mips mips (clk, rst, 0, instr, rd_dm, we_dm, pc_current, alu_out, wd_dm, DONT_USE);

imem imem (pc_current[7:2], instr);

dmem dmem (clk, we_dm, alu_out[7:2], wd_dm, rd_dm);

endmodule

module mips

(input clk, rst, [4:0] ra3, [31:0] instr, rd_dm, output we_dm, [31:0] pc_current, alu_out, wd_dm, rd3);

wire       pc_src, jump, we_reg, alu_src, JReturn, writeHilo, setJal;

wire [2:0] alu_ctrl;

wire [1:0] reg_dst, dm2reg;

datapath    dp (clk, rst, pc_src, jump, JReturn, writeHilo, setJal, we_reg, alu_src, dm2reg, reg_dst, alu_ctrl, ra3, instr, rd_dm, zero, pc_current, alu_out, wd_dm, rd3);

controlunit cu (zero, instr[31:26], instr[5:0], pc_src, jump, we_reg, alu_src, we_dm, JReturn, writeHilo, setJal, dm2reg, reg_dst, alu_ctrl);

endmodule

module mips_fpga

(input clk, rst, button, [7:0] switches, output we_dm, sink_bit, [7:0] LEDSEL, LEDOUT);

reg  [15:0] reg_hex;

wire        clk_sec, clk_5KHz, clk_pb;

wire [7:0]  digit0, digit1, digit2, digit3, digit4, digit5, digit6, digit7;

wire [31:0] pc_current, instr, alu_out, wd_dm, rd_dm, dispData;

clk_gen     clk_gen (clk, rst, clk_sec, clk_5KHz);

bdebouncer  bd      (clk_5KHz, button, clk_pb);

mips        mips    (clk_pb, rst, switches[4:0], instr, rd_dm, we_dm, pc_current, alu_out, wd_dm, dispData);

/*

switchs[4:0] are used as the 3rd read address (ra3) of the RF,

dispData is the register contents from the RF's 3rd read port (rd3).

*/

imem        imem    (pc_current[7:2], instr);

dmem        dmem    (clk_pb, we_dm, alu_out[7:2], wd_dm, rd_dm);

bcd_to_7seg bcd7    (pc_current[15:12], digit7);

bcd_to_7seg bcd6    (pc_current[11:8], digit6);

bcd_to_7seg bcd5    (pc_current[7:4], digit5);

bcd_to_7seg bcd4    (pc_current[3:0], digit4);

bcd_to_7seg bcd3    (reg_hex[15:12], digit3);

bcd_to_7seg bcd2    (reg_hex[11:8], digit2);

bcd_to_7seg bcd1    (reg_hex[7:4], digit1);

bcd_to_7seg bcd0    (reg_hex[3:0], digit0);

led_mux     led_mux (clk_5KHz, rst, digit7, digit6, digit5, digit4, digit3, digit2, digit1, digit0, LEDSEL, LEDOUT);

/*

switches[7:5] = 000 : Display lower half word of register selected by switches[4:0]

switches[7:5] = 001 : Display higher half word of register selected by switches[4:0]

switches[7:5] = 010 : Display lower half word of 'instr'

switches[7:5] = 011 : Display higher half word of 'instr'

switches[7:5] = 100 : Display lower half word of 'alu_out'

switches[7:5] = 101 : Display higher half word of 'alu_out'

switches[7:5] = 110 : Display lower half word of 'wd_dm'

switches[7:5] = 111 : Display higher half word of 'wd_dm'

*/

always @ (posedge clk)

begin

case ({switches[7], switches[6], switches[5]})

3'b000:	reg_hex = dispData[15:0];

3'b001: reg_hex = dispData[31:16];

3'b010: reg_hex = instr[15:0];

3'b011: reg_hex = instr[31:16];

3'b100: reg_hex = alu_out[15:0];

3'b101: reg_hex = alu_out[31:16];

3'b110: reg_hex = wd_dm[15:0];

3'b111: reg_hex = wd_dm[31:16];

endcase

end

assign sink_bit = (pc_current > 0) ^ (instr > 0) ^ (alu_out > 0);

endmodule