# Factorial Calculator:

# Introduction:

The purpose of this lab was to create a factorial calculator using Xilinx Vivado Design Suite through Verilog programming. This module takes a 4-bit binary value and outputs the factorial on a 7-segment display. The module is only able to process factorials as high as 12!, and will give an error message if any the input is a larger number than 12.

The list of tasks were fully completed in lab:

-       Waveform validation of factorial

-       Design system datapath using basic building blocks

-       Design block diagrams

-       Construct output table

-       Hardware validation using Nexys4 DDR board

-       Stimuli and Hardware Verification of  factorial

The list of tasks were not fully completed in lab but were finished afterword:

-       Simulation and hardware verification of the error flag

# Design Methodology:

The lab assignment called for a hierarchical design consisting of several modules:

## factorial_top:

This module takes a 4-bit input, a go signal, and a reset signal, and outputs a done signal, a 32-bit product, and an error flag. The input is processed by the datapath while the go signal is processed by the control unit. The product is generated by the datapath while the done and error flags are generated by the control unit.

## control_unit:

This module takes in go and reset signals from the top module, as well a greater than signal, and a greater than 12 signal from the datapath to generate next state logic. The control unit has 5 states. State 0 is an idle state waiting for the command to begin. State 1 prepares the datapath to begin calculating the factorial. State 2 is a wait state for the internal multiplier to finish its calculations. State 3 is the done state where an output enable is sent to the datapath and the done and error flags will be read. State 4 is a transition state where counter and register values of the datapath are updated.

## datapath:

This module takes a 4 bit input from the top module, as well as load counter, enable counter, load register, and output enable signals from the control unit, and outputs a 32-bit product to the top module as well as a greater than signal and a greater than 12 signal to the control unit.

# Procedure:

- Create factorial project in Vivado and select the proper hardware: xc7a100csg324-1.

- Create factorial_top.v, control_unit.v, factorial_dp.v, and self-checking testbenches factorial_top_tb.v, control_unit_tb.v, and factorial_dp_tb.v.

- Run simulations and verify functionality performs as desired.

- Create and design factorial_FPGA.v, factorial_FPGA.xdc,  and all modules necessary for board functionality.

- Run Synthesis, Implementation, and Bitstream Generation, and program the Nexys 4 FPGA board.

- Verify board Functionality is performing as desired.

# Simulation Results:

Figure 1a: factorial_top_tb.v simulation result.

Figure 1b: factorial_top_tb.v Tcl console output.

Figure 2a: control_unit_tb.v simulation results.

Figure 2b: control_unit_tb.v Tcl console output.

Figure 3a: datapath_tb.v simulation results.

Figure 3b: datapath_tb.v Tcl console output.

# FPGA Validation:

For hardware validation, shown in the pictures below of input 7 and o which started in state 0 with Go flag set. The results are shown in decimal with the output of 5040, which is correct for 7!, and output 1 which is correct for 0!.

Figure 4a: FPGA Validation Result of Factorial 0.

Figure 4b: FPGA Validation Result of Factorial 7.

# Conclusion:

All tasks outlined in the Procedure section were successful. Conflict originally arose when implementing the top-level design on the FPGA board, but this was rectified and board functionality was completed as intended. Proper functionality of source code was also verified by simulations. This lab was a good test of retained knowledge from prerequisite courses.

# Appendix:

## Diagrams:

Figure 5: Top Module Block Diagram.

Figure 6: Control Unit Finite State Machine.

Figure 7: Data Path Module.

Figure 8: State Transition Diagram.

Figure 9: Next State Logic Diagram.

| CS | go | load_cnt | load_reg | cnt_en | sel1 | sel2 | ud | GT | NS | DONE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | x | x | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | x | x | 1 | 0 |
| 1 | x | 1 | 1 | 1 | 1 | 1 | x | x | 2 | 0 |
| 2 | x | 0 | 0 | 1 | 0 | 1 | x | 0 | 4 | 0 |
| 2 | x | 0 | 0 | 1 | 0 | 1 | x | 1 | 3 | 0 |
| 3 | x | 0 | 1 | 1 | 0 | 0 | 0 | x | 2 | 0 |
| 4 | x | 0 | 0 | 0 | 0 | 0 | x | x | 0 | 1 |

Table 1: State Output Table

## Source Code:

factorial_top.v

`timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: // Design Name: // Module Name: Factorial // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module Factorial(input [3:0] n, input Go, clk, rst, output Done, output [31:0] out); wire [5:0] ctrl; wire Is_Gt; Factorial_CU U0 (.Go(Go), .clk(clk), .rst(rst), .Is_Gt(Is_Gt), .Done(Done), .ctrl(ctrl)); Factorial_DP U1 ( .n(n), .ctrl(ctrl), .clk(clk), .rst(rst), .out(out), .Is_GT(Is_Gt)); endmodule

control_unit.v

`timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: // Design Name: // Module Name: Factorial_CU // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module Factorial_CU(input Go, clk, rst, Is_Gt, output reg Done, output reg [5:0] ctrl); reg [3:0] ns, cs; parameter IDLE = 6'b0_0_0_0_0_0, S1 = 6'b0_1_0_1_1_0, S2 = 6'b1_0_0_0_0_0, S3 = 6'b1_0_0_0_0_1, S4 = 6'b1_1_0_0_1_0; always @(Go, Is_Gt, Done, ns, cs) begin case(cs) 4'd0: begin Done = 0; if(Go) begin ns = 4'd1; end else begin ns = 4'd0; end end 4'd1: begin Done = 0; ns = 4'd2; end 4'd2: begin if(Is_Gt) begin ns = 4'd4; Done = 0; end else begin ns = 4'd3; Done = 0; end end 4'd3: begin Done = 1; ns = 4'd0; end 4'd4: begin Done = 0; ns = 4'd2; end default: ns = 4'd0; endcase end always @(posedge clk, posedge rst) begin if(rst) cs <= 0; else cs <= ns; end always @(cs, ctrl) //{Sel1, ce, ud, ld1, ld2, sel2} = ctrl; begin case(cs) 4'd0: ctrl = IDLE; 4'd1: ctrl = S1; 4'd2: ctrl = S2; 4'd3: ctrl = S3; 4'd4: ctrl = S4; endcase end endmodule

datapath.v

`timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: // Design Name: // Module Name: Factorial_DP // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module Factorial_DP(input [3:0] n, input [5:0] ctrl, input clk, rst, output [31:0] out, output Is_GT); wire [3:0] Cnt_Reg; wire [31:0] q_out, aluout, q2_out, mux_out1, mux_out2; wire IS_equal; Ud_Cnt_4 u1(.D(n), .LD(ctrl[2]), .UD(ctrl[3]), .CE(ctrl[4]), .CLK(clk), .RST(rst), .Q(Cnt_Reg)); comparater u2(.A({28'd0,Cnt_Reg}), .B(32'd1), .greater(Is_GT)); Dreg u3(.D(mux_out1), .en(ctrl[1]), .clk(clk), .rst(rst), .Q(q_out)); Alu u4(.in1(q_out), .in2({28'd0,Cnt_Reg}), .c(2'b00), .aluout(aluout)); Mux u6 (.in1(32'd1), .in2(aluout), .sel(ctrl[5]), .out(mux_out1)); Mux u7 (.in1(32'd0), .in2(aluout), .sel(ctrl[0]), .out(out)); endmodule

factorial_FPGA.v

`timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: // Design Name: // Module Name: Factorial_fpga // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module Factorial_fpga(input [3:0] n, input go, clk100MHz, control, rst, output Done, output [7:0] LEDOUT, LEDSEL ); supply1 [7:0] vcc; wire DONT_USE, clk_5KHz; wire [31:0] out; wire [3:0] dig0, dig1, dig2, dig3, dig4, dig5, dig6, dig7; wire [6:0] out0, out1, out2, out3, out4, out5, out6, out7; Factorial U0 (.n(n), .Go(go), .clk(debouncedButton), .rst(rst), .Done(Done), .out(out)); bin2bcd32 U1 (.value(out), .dig0(dig0), .dig1(dig1), .dig2(dig2), .dig3(dig3), .dig4(dig4), .dig5(dig5), .dig6(dig6), .dig7(dig7)); showNumber U2 (.dig0(dig0), .dig1(dig1), .dig2(dig2), .dig3(dig3), .dig4(dig4), .dig5(dig5), .dig6(dig6), .dig7(dig7), .out0(out0), .out1(out1), .out2(out2), .out3(out3), .out4(out4), .out5(out5), .out6(out6), .out7(out7)); led_mux U3(clk_5KHz, rst, {1'b1, out7}, {1'b1, out6}, {1'b1, out5}, {1'b1, out4}, {1'b1, out3}, {1'b1, out2}, {1'b1, out1},{1'b1, out0}, LEDOUT, LEDSEL); clk_gen U4(.clk100MHz(clk100MHz), .rst(rst), .clk_4sec(DONT_USE), .clk_5KHz(clk_5KHz)); button_debouncer U5(.clk(clk_5KHz), .button(control), .debounced_button(debouncedButton)); endmodule

factorial_FPGA.xdc

set_property -dict { PACKAGE_PIN E3 IOSTANDARD LVCMOS33 } [get_ports { clk100MHz }]; create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports {clk100MHz}]; set_property -dict { PACKAGE_PIN J15 IOSTANDARD LVCMOS33 } [get_ports { n[0] }]; set_property -dict { PACKAGE_PIN L16 IOSTANDARD LVCMOS33 } [get_ports { n[1] }]; set_property -dict { PACKAGE_PIN M13 IOSTANDARD LVCMOS33 } [get_ports { n[2] }]; set_property -dict { PACKAGE_PIN R15 IOSTANDARD LVCMOS33 } [get_ports { n[3] }]; set_property -dict { PACKAGE_PIN H17 IOSTANDARD LVCMOS33 } [get_ports { Done }]; set_property -dict { PACKAGE_PIN M17 IOSTANDARD LVCMOS33 } [get_ports { rst }]; set_property -dict { PACKAGE_PIN N17 IOSTANDARD LVCMOS33 } [get_ports { control }]; set_property -dict { PACKAGE_PIN R17 IOSTANDARD LVCMOS33 } [get_ports { go }]; set_property -dict { PACKAGE_PIN K13 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[0] }]; set_property -dict { PACKAGE_PIN K16 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[1] }]; set_property -dict { PACKAGE_PIN P15 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[2] }]; set_property -dict { PACKAGE_PIN L18 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[3] }]; set_property -dict { PACKAGE_PIN R10 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[4] }]; set_property -dict { PACKAGE_PIN T11 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[5] }]; set_property -dict { PACKAGE_PIN T10 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[6] }]; set_property -dict { PACKAGE_PIN H15 IOSTANDARD LVCMOS33 } [get_ports { LEDOUT[7] }]; set_property -dict { PACKAGE_PIN J17 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[0] }]; set_property -dict { PACKAGE_PIN J18 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[1] }]; set_property -dict { PACKAGE_PIN T9 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[2] }]; set_property -dict { PACKAGE_PIN J14 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[3] }]; set_property -dict { PACKAGE_PIN P14 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[4] }]; set_property -dict { PACKAGE_PIN T14 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[5] }]; set_property -dict { PACKAGE_PIN K2 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[6] }]; set_property -dict { PACKAGE_PIN U13 IOSTANDARD LVCMOS33 } [get_ports { LEDSEL[7] }];

datapath_tb.v

`timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: // Design Name: // Module Name: dp_tb // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module dp_tb(); reg [3:0] n_tb; reg [5:0] ctrl_tb; reg clk_tb, rst_tb; wire [31:0] out_tb; wire Is_GT_tb; integer i; Factorial_DP DUT( .n(n_tb), .ctrl(ctrl_tb), .clk(clk_tb), .rst(rst_tb), .out(out_tb), .Is_GT(Is_GT_tb)); parameter IDLE = 6'b0_0_0_0_0_0, S1 = 6'b0_1_0_1_1_0, S2 = 6'b1_0_0_0_0_0, S3 = 6'b1_0_0_0_0_1, S4 = 6'b1_1_0_0_1_0; task tick; begin #5 clk_tb = 1; #5 clk_tb = 0; end endtask initial begin clk_tb = 0; rst_tb = 0; for(i = 0; i < 13; i = i + 1) begin n_tb = i; ctrl_tb = IDLE; tick; ctrl_tb = S1; tick; while(Is_GT_tb) begin ctrl_tb = S2; tick; ctrl_tb = S4; tick; end ctrl_tb = S3; tick; case(i) 4'd0: begin if(out_tb != 0) $display("Error with 0"); end 4'd1: begin if(out_tb != 1) $display("Error with 1"); end 4'd2: begin if(out_tb != 2) $display("Error with 2"); end 4'd3: begin if(out_tb != 6) $display("Error with 3"); end 4'd4: begin if(out_tb != 24) $display("Error with 4"); end 4'd5: begin if(out_tb != 120) $display("Error with 5"); end 4'd6: begin if(out_tb != 720) $display("Error with 6"); end 4'd7: begin if(out_tb != 5040) $display("Error with 7"); end 4'd8: begin if(out_tb != 40320) $display("Error with 8"); end 4'd9: begin if(out_tb != 362880) $display("Error with 9"); end 4'd10: begin if(out_tb != 3628800) $display("Error with 10"); end endcase end $display("Sucess"); $finish; end endmodule

cu_tb.v

`timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: // Design Name: // Module Name: cu_tb // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module cu_tb(); reg Go_tb, clk_tb, rst_tb, Is_Gt_tb; wire Done_tb; wire [5:0] ctrl_tb; Factorial_CU DUT(.Go(Go_tb), .clk(clk_tb), .rst(rst_tb), .Is_Gt(Is_Gt_tb), .Done(Done_tb), .ctrl(ctrl_tb)); parameter IDLE = 6'b0_0_0_0_0_0, S1 = 6'b0_1_0_1_1_0, S2 = 6'b1_0_0_0_0_0, S3 = 6'b1_0_0_0_0_1, S4 = 6'b1_1_0_0_1_0; task tick; begin #5 clk_tb = 1; #5 clk_tb = 0; end endtask initial begin clk_tb = 0; rst_tb = 0; Go_tb = 1; Is_Gt_tb = 1; tick; if(ctrl_tb != IDLE) $display("Error with IDLE"); tick; if(ctrl_tb != S1) $display("Error with S1"); tick; if(ctrl_tb != S2) $display("Error with S2"); tick; if(ctrl_tb != S4) $display("Error with S4"); tick; Is_Gt_tb = 0; tick; if(ctrl_tb != S3) begin $display("Error with S3"); if(Done_tb != 1) $display("Error"); end $display("Success"); $finish; end endmodule

factorial_top_tb.v

`timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: // Design Name: // Module Name: Factorial_tb // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module Factorial_tb(); reg [3:0] n_tb; reg Go_tb, clk_tb, rst_tb; wire Done_tb; wire [31:0] out_tb; integer i; Factorial DUT (.n(n_tb), .Go(Go_tb), .clk(clk_tb), .rst(rst_tb),.Done(Done_tb), .out(out_tb)); task tick; begin #5 clk_tb = 1; #5 clk_tb = 0; end endtask initial begin n_tb = 0; clk_tb = 0; rst_tb = 0; Go_tb = 0; tick; tick; tick; tick; tick; tick; tick; tick; tick; tick; Go_tb = 1; for(i = 0; i < 11; i = i + 1) begin n_tb = i; tick; while(!Done_tb) begin tick; end case(i) 4'd0: begin if(out_tb != 1) $display("Error with 0"); end 4'd1: begin if(out_tb != 1) $display("Error with 1"); end 4'd2: begin if(out_tb != 2) $display("Error with 2"); end 4'd3: begin if(out_tb != 6) $display("Error with 3"); end 4'd4: begin if(out_tb != 24) $display("Error with 4"); end 4'd5: begin if(out_tb != 120) $display("Error with 5"); end 4'd6: begin if(out_tb != 720) $display("Error with 6"); end 4'd7: begin if(out_tb != 5040) $display("Error with 7"); end 4'd8: begin if(out_tb != 40320) $display("Error with 8"); end 4'd9: begin if(out_tb != 362880) $display("Error with 9"); end 4'd10: begin if(out_tb != 3628800) $display("Error with 10"); end endcase end $display("Success"); $finish; end endmodule