Introduction

The lab consists of four major parts; designing a carry look adder (CLA), an inferred adder, writing test benches for both design as verification to the design, and writing the hardware validating software to be used on the Nexys board. The CLA design segments use a hierarchical approach where designs are built using smaller components and combined in the end at the highest level. The inferred adder takes on a more direct approach in which pre defined operators are used making the design less complex as Xilinx software handles all of the operations for the user. A singular test bench was written to validate the designs through simulations. The test bench can be applied to either the CLA or inferred adder. Lastly the designs were validated with a physical device which utilized LEDs, switches and the seven segment display.

The CLA design expected was to utilize a strict structure in which the design was to be broken down. The building blocks consists of half adders, glue logic, and a custom handling BCD splitter. This component was presented in lab but the design did not follow the design requirements. Missing parts such as the splitter were missing. The design presented was the same as the design expected which in result was able to achieve the proper stimuli. However the design presented used hierarchical design which utilized the one half adder instead of four half adders. The singular adder was utilized four times and summed at the end.

The parts listed below were not fully completed in lab but were finished afterword:

- Design 4-bit inferred adder

- Verify 4-bit inferred adder

- Verify both ALU and 4-bit inferred adders using switches, LEDs, and the seven segment display

Design Methodology

Carry Look Ahead Adder (CLA): CLA design is split into many modules. The first module consists of multiple half adders which took in inputs and and outputs were sent to the CLA generator. The sums were calculated using another module. The top level module was used as a binder to utilize other smaller modules. This ensures every module can be testing separately and or in large set.  Figure one shows the design of the custom CLA adder implemented.

Figure 1. CLA Block Diagram

The basic starting point was a single half adder as shown below in figure two. Four of these adders created a set of quad adders which was designed in a separate submodule. All adders were used to perform a summing operation which would result in a carry out output. This resulted in a carry-in input for the other adders within the submodule. Spawning a new sub-module to handle the carry-in and carry-out bits. The finalized output sum was calculated by a multiple XORs and  AND’ed carry-in input when needed. Below is a truth table which represents a full adder which is used for validation explained in the next section.

Table 1: Full Adder Truth Table

| carry-ini | ai ⊕ bi | ai & bi | si= ai+bi+ci | carry-ini+1 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 (ai⊕bi⊕ci) | 0 ((ai⊕bi)ci+ ai & bi) |
| 0 | 1 | 0 | 1 (ai⊕bi⊕ci) | 0 ((ai⊕bi)ci+ ai & bi) |
| 0 | 1 | 0 | 1 (ai⊕bi⊕ci) | 0 ((ai⊕bi)ci+ ai & bi) |
| 0 | 0 | 1 | 0 (ai⊕bi⊕ci) | 1 ((ai⊕bi)ci+ ai & bi) |
| 1 | 0 | 0 | 1 (ai⊕bi⊕ci) | 0 ((ai⊕bi)ci+ ai & bi) |
| 1 | 1 | 0 | 0 (ai⊕bi⊕ci) | 1 ((ai⊕bi)ci+ ai & bi) |
| 1 | 1 | 0 | 0 (ai⊕bi⊕ci) | 1 ((ai⊕bi)ci+ ai & bi) |
| 1 | 0 | 1 | 1 (ai⊕bi⊕ci) | 1 ((ai⊕bi)ci+ ai & bi) |

Figure 2. Singular Half Adder Block Diagram

Figure 3. Carry Look-Ahead Generator block diagram

Figure 4. XOR block diagram

The inferred adder performs the same function as the custom CLA adder. However instead of defining modules to perform the summing operation the built in addition operator was utilized. This design is much more abstract and but results in the same output. The design for this adder is much less complex and easier to implement. The logical flow is coherent to the custom CLA in which the half adders are summed to create a full adder. Figure three shows the block diagram for the inferred adder.

Figure 5. Inferred Adder Block Diagram

Simulation Test Plan

A single self checking testbench stimuli was used to validate both adders. Due to the logic and operational characteristics being the same for both adders the test bench implemented loops, if statements, and comparison logic to generate multiple stimuli. The testbench defined its 4-bit inputs plus a carry-in and expected a proper output plus a carry-out. The first section of the testbench sets the carry-in as 0 and goes through all possible inputs. The second sections sets the carry-in to 1 and loops over all inputs. Within the loops if statements check to see if the sum is equivalent to the output. For a successful testbench run for both the CLA and inferred adder the console output is shown in figure four and figure five. Additional waveforms can be found in the appendix.

Figure 4. Console output for Adder

Figure 5. Console output for Adder

FPGA Test Plan

To attempt to validate the board, three new modules were added to be able to use the seven segment display. The XDC file was set to use the switches, RGB LED and seven segment display. However we were unable to get this part to work as the bitstream was failing.

Conclusion

In conclusion, our carry look ahead adder performed as expected, however, it did not meet the requirements for a hierarchical design approach. We have since rectified this and properly designed the lab reflecting the modular design. In addition to fixing the carry look ahead design, we designed and successfully tested the inferred adder. We were unable to successfully test the FPGA board and could not verify the complete functionality of the board.

Appendix

Figure x. Additional Waveforms for Stimuli

Source Code

Files for Presented Design

CLA_ADD.v

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// wiresompany:

// Engineer:

//

// wiresreate Date: 03/08/2017 04:26:38 pM

// Design Name:

// Module Name: wiresLA_ADD

// projewirest Name:

// Target Deviwireses:

// Tool Versions:

// Deswiresription:

//

// Dependenwiresies:

//

// Revision:

// Revision 0.01 - File wiresreated

// Additional wiresomments:

//

//////////////////////////////////////////////////////////////////////////////////

module half_adder(A,B,p,g);

input A;

input B;

output p; //and p=AB

output g;//XOR g=A^B

assign p=A^B;

assign g=A&B;

endmodule

module half_adder2(p,g,wiresin,Sum,wiresout);

input [3:0] p;

input [3:0] g;

input wiresin;

output [3:0] Sum;

output wiresout;

wire [3:0] wires;

assign wires[0]=wiresin;

assign wires[1] = g[0] | (p[0] & wires[0]);

assign wires[2] = g[1] | (p[1] & g[0]) | (p[1] & p[0] & wires[0]);

assign wires[3] = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & wires[0]);

assign wiresout = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]) |(p[3] & p[2] & p[1] & p[0] & wires[0]);

assign wires[4] = wiresout;

assign Sum =p^wires; //sum all wires i hope

endmodule

module top(A,B,Sum,wiresout,wiresin);

input [3:0] A;

input [3:0] B;

input wiresin;

output wiresout;

output [3:0] Sum;

wire [3:0] p;

wire [3:0] g;

half_adder DUT1(.p(p[0]),.g(g[0]),.A(A[0]),.B(B[0]));

half_adder DUT2(.p(p[1]),.g(g[1]),.A(A[1]),.B(B[1]));

half_adder DUT3(.p(p[2]),.g(g[2]),.A(A[2]),.B(B[2]));

half_adder DUT4(.p(p[3]),.g(g[3]),.A(A[3]),.B(B[3]));

// wiresall other modules to sum them up

half_adder2 DUT5(.p(p),.g(g),.Sum(Sum),.wiresout(wiresout),.wiresin(wiresin));

endmodule

TB_CLAADD.v

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// wiresompany:

// Engineer:

//

// wiresreate Date: 03/08/2017 04:34:34 pM

// Design Name:

// Module Name: TB_wiresLAADD

// projewirest Name:

// Target Deviwireses:

// Tool Versions:

// Deswiresription:

//

// Dependenwiresies:

//

// Revision:

// Revision 0.01 - File wiresreated

// Additional wiresomments:

//

//////////////////////////////////////////////////////////////////////////////////

module TB_wiresLAADD;

reg [3:0]tb_A;

reg [3:0]tb_B;

reg tb_in;

wire [3:0] tb_sum;

wire tb_wiresout;

reg [3:0] expewirested_input;

reg tb_wiresout_expewirested;

integer A_inputs;

integer B_inputs;

top DUT(.A(tb_A), .B(tb_B),.Sum(tb_sum),.wiresout(tb_wiresout),.wiresin(tb_in));

//inferred DUT2 (.A(tb_A), .B(tb_B), .Sum(tb_sum), .wiresout(tb_wiresout),.wiresin(tb_in));

initial

begin

tb_in=0;

for(A_inputs=0;A_inputs<16;A_inputs=A_inputs+1)

begin

tb_A=A_inputs;

for(B_inputs=0;B_inputs<16;B_inputs=B_inputs+1)

begin

tb_B=B_inputs;

expewirested_input=tb_A+tb_B;

#3;

if(tb_A+tb_B> 15)

begin

tb_wiresout_expewirested=1;

#3;

end

else

begin

tb_wiresout_expewirested=0;

#5;

end

if((tb_sum!=expewirested_input)||(tb_wiresout_expewirested!=tb_wiresout))

begin

$display("Error broken");

$stop;

end

end

end

tb_in=1;

for(A_inputs=0;A_inputs<16;A_inputs=A_inputs+1)

begin

tb_A=A_inputs;

for(B_inputs=0;B_inputs<16;B_inputs=B_inputs+1)

begin

tb_B=B_inputs;

expewirested_input=((tb_A+tb_B)+ 4'b0001);

if(tb_A+tb_B+1>15)

begin

tb_wiresout_expewirested=1;

#5;

end

else

begin

tb_wiresout_expewirested=0;

#5;

end

if((tb_sum!=expewirested_input)||(tb_wiresout_expewirested!=tb_wiresout))

begin

$display("Error broken");

$stop;

end

end

end

$display("Adder was succesful!");

$stop;

end

endmodule

Files added for Corrected Design

generater.v

module GENERATOR(

input [3:0] P, G, input Cin,

output reg [3:0] C, output reg Cout

);

//Ci = Gi | Pi & Ci;

always @(P, Q, Cin, C) begin

C[0] = Cin;

C[1] = G[0] | P[0] & C[0];

C[2] = G[1] | P[1] & C[1];

C[3] = G[2] | P[2] & C[2];

Cout = G[3] | P[3] & C[3];

end

endmodule

module SUMMER(

input [3:0] P, C,

output reg [3:0] Sum

);

xor U0(Sum[0], P[0], C[0]);

xor U1(Sum[1], P[1], C[1]);

xor U2(Sum[2], P[2], C[2]);

xor U3(Sum[3], P[3], C[3]);

endmodule

CLA_ADD.v

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// wiresompany:

// Engineer:

//

// wiresreate Date: 03/08/2017 04:26:38 pM

// Design Name:

// Module Name: wiresLA_ADD

// projewirest Name:

// Target Deviwireses:

// Tool Versions:

// Deswiresription:

//

// Dependenwiresies:

//

// Revision:

// Revision 0.01 - File wiresreated

// Additional wiresomments:

//

//////////////////////////////////////////////////////////////////////////////////

module half_adder(A,B,p,g);

input A;

input B;

output p; //and p=AB

output g;//XOR g=A^B

assign p=A^B;

assign g=A&B;

endmodule

module half_adder2(p,g,wiresin,Sum,wiresout);

input [3:0] p;

input [3:0] g;

input wiresin;

output [3:0] Sum;

output wiresout;

wire [3:0] wires;

assign wires[0]=wiresin;

assign wires[1] = g[0] | (p[0] & wires[0]);

assign wires[2] = g[1] | (p[1] & g[0]) | (p[1] & p[0] & wires[0]);

assign wires[3] = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & wires[0]);

assign wiresout = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]) |(p[3] & p[2] & p[1] & p[0] & wires[0]);

assign wires[4] = wiresout;

assign Sum =p^wires; //sum all wires i hope

endmodule

GENERATOR U1(

.P(P_wire), .G(G_wire), .Cin(Cin_top),

.C(C_wire), .wiresout(wiresout)

);

SUMMER U2(

.P(P_wire), .C(C_wire),

.Sum(Sun)

);

module top(A,B,Sum,wiresout,wiresin);

input [3:0] A;

input [3:0] B;

input wiresin;

output wiresout;

output [3:0] Sum;

wire [3:0] p;

wire [3:0] g;

half_adder DUT1(.p(p[0]),.g(g[0]),.A(A[0]),.B(B[0]));

half_adder DUT2(.p(p[1]),.g(g[1]),.A(A[1]),.B(B[1]));

half_adder DUT3(.p(p[2]),.g(g[2]),.A(A[2]),.B(B[2]));

half_adder DUT4(.p(p[3]),.g(g[3]),.A(A[3]),.B(B[3]));

// wiresall other modules to sum them up

half_adder2 DUT5(.p(p),.g(g),.Sum(Sum),.wiresout(wiresout),.wiresin(wiresin));

inferred.v

module inferred(A,B,wiresin, wiresout, Sum);

input [3:0] A;

input [3:0] B;

input Carry_in;

output wiresout;

output [3:0] Sum;

assign {wiresout,Sum}=A+B+wiresin;

endmodule